Soil Moisture Sensor Specifications

For more info check: Hrisko (2020) Capacitive Soil Moisture Sensor Theory, Calibration, and Testing. DOI: 10.13140/RG.2.2.36214.83522

- Material used:

- Probe#1: Capacitive soil moisture sensor v2.0; HW-390; 3,3..5 V; length 10cm; width 2.2cm;

- Arduino UNO Microcontroller ATmega328 R3 Board CH340; Boardversion R3; USB-Chip: CH340; Microcontroller Atmel ATmega328-PU (SMD-Version); 14 digital GPIOs (6 with PWM-Funktion); 6 analogue inputs; Output Voltage 3.3V, 5V; Recommended Input Voltage 7-12V; (max) input voltage 6-20V; 1KB EEPROM; 32KB Flash, 0.5KB Bootloader

- Arduino Compatible Micro SD Storage Board; 5V or 3.3V; power supply is 4.5V ~ 5.5V, 3.3V voltage regulator circuit board; SPI interface; six pins (GND, VCC, MISO, MOSI, SCK, CS)

- Cables

- Connections:

Probe#1 to control board:

- Module GND -- Arduino GND pin

- Module VCC -- Arduino VCC pin

- Module AUOT -- Arduino A0 pin

SD Card to control board:

- Calibrating and Testing:

Calibration Readings:

In full dry conditions (air): 510 units

In full dry conditions (dry soil) = 9%  18.86 (humidity)  482 (dryness) (USED)

In full wet conditions (glass of water): 192 units

In full wet conditions (Saturated soil) = 98%  66.92 (humidity)  196 (dryness) (USED)

- On-Site-Procedure:

- Recording geolocation of measurement point.

- Starting SM sensor in dry air for control.

- Removing topsoil layer with dense organic matter.

- Inserting SM sensor at 10 cm depth.

- Keeping SM sensor stable for at least 1 min.

- Removing sensor from soil.

- Waiting 30 seconds in dry air for control.

- Turning off sensor.

The average of readings was calculated at each point after sensor stabilized. The readings of soil moisture (humidity) was converted to Plant Available Water (PAW).

- PAW (%) is calculated: PAW = 100x (SM – WP)/(FC –WP)

- For loam soil: FC=33% and WP=13%